clockprobe, a pixel clock probing program

=========================================

This program tries to measure the CURRENT pixel clock. This means it does

NOT change clocks like the X-server does. It also implies that it should

work on ANY VGA card, since it only uses standard VGA registers.

The use of this program should be obvious, and manyfold:

- if you want to "grab" a standard VGA text mode for inclusion in your own

TextConfig (or XF86Config) file, you'd probably use "vgaset". But vgaset

does NOT tell you the pixel clock currently used. This program can tell

you that missing piece of information.

- if using SVGATextMode fails to show you the kind of screen you'd expect,

and/or your monitor doesn't seem to like what you send it, then clockprobe

(even when typed blindly, redirected to a file for later viewing) should

be able to tell you what clock you are ACTUALLY using.

It's not because your "clocks" line says "50 MHz" for some clock, that

that clock index actually IS 50 MHz. You could be using the wrong clocks

line, or there could be a bug in the clock setting code.

In either case, clockprobe can tell you the ACTUAL clock.

- ...

Usage:

../clockprobe: version 0.9. (c) 1995 Koen Gadeyne.

Usage: ../clockprobe [options]

Options: -n  Don't program VGA hardware

-d  print debugging information

-h  print usage information

Normally you use clockprobe without any parameters.

Here's a sample output from clockprobe:

> clockprobe

clockprobe: Estimated vertical scanrate = 70.336 Hz.

clockprobe: Estimated horizontal scanrate = 56.269 kHz.

clockprobe: Estimated pixel clock (for non-interlaced display) = 75.18 MHz

Clockprobe will make no distinction between interlaced or non-interlaced

displays. The vertical frequency given will be the REAL vertical frequency.

That means that if the previous example were to be interlaced, the result

would have been:

clockprobe: Estimated vertical scanrate = 140.672 Hz.

clockprobe: Estimated horizontal scanrate = 56.269 kHz.

clockprobe: Estimated pixel clock (for non-interlaced display) = 75.18 MHz

Only the vertical frequency differs: it is double the non-interlaced

frequency.

It would be an interesting feature of clockprobe to mention the fact if the

mode is interlaced or not together with the vertical frequency.

But that would make this program VGA-card dependent, since interlacing is

implemented differently on every other VGA card. Just like clock selection.

There HAS to be a way around this...

----------------------------------------------------------------------

Known limitations:

------------------

This program is my first attempt at clock probing, and it works on MY

machine. It might very well NOT work for you. Test it on a known clock

before attempting any others.

Also note the accuracy is best when the system is not severely loaded.

Especially disk access seems to have a bad influence.

clockprobe uses a derivative of a statistical method called "histogram

windowing", throwing away measurements that look out-of-bounds. It performs

pretty well, even on busy machines. If too many measurements are thrown

away, it will do some retries, hoping for better results. After a few

retries, it will output the results from the latest attempt, warning you

about the low accuracy you can expect.

One small "bug" seems to be that it always over-estimates the clock by

approx 0.1 to 0.2 MHz on my machine. This could either be caused by bad

programming, an inaccurate hardware timer, a kernel problem, or something

else I couldn't think of right now. Hopefully someone out there will be able

to tell me what the cause of this is, or that it works perfectly on his

machine :-)

But that should not really be a problem, since accuracy is good enough for

what it's intended for: determining if you got the right clock, or finding

out what clock index you're using. Most VGA cards don't have clocks that lie

within a few MHz from each other (there's no use for that anyway), so you

should be able to find out WHAT clock index you are using.

Another possible problem could be that it might not work properly on "slow"

machines (386...). Mine is a 486-DX40, and it works fine. Slow boxes might

run out of breath during measurement, giving completely erroneous results,

or failing to find enough "valid" measurements to get a trustworthy result.

And finally: clockprobe detects the use of the "divide by 2" feature, and

reports that the actual clock is double the one probed. This feature is

still standard VGA, but some cards (Trident, ET4000, ...) have a built-in

extra divider (ET4000: by 2 and 4; Trident: 1.5, 2 and 4). Since this

program does not WANT to be card-dependent, it cannot detect that. So it

will simply tell you what clock is actually used to pump pixels to the

display, while the ACTUAL pixel clock COULD be a multiple of that. Anyway,

the really interesting thing is the one used (= possibly divided), so this

should not be a real problem.

Some interesting features of this program are:

- No need to disable interrupts to be able to measure! (the probe in the

X-server does, because it doesn't use timers).

- Even busy machines should give a reasonable result, unlike the X-probe,

which needs the entire machine for itself (i.e. interrupts disabled, no

task switches). Even heavily loaded (swapping!) machines should give a

rather close estimate (maybe a few MHz off, but that should be all).  I

was able to run 2 fli-animations in X-windows, a "cat< </dev/zero

>/dev/null" and a compilation all together on a 16 Meg machine., while

probing for the current clock. The result was only 1 MHz form where it

should be. During measurement, the machine was doing LOTS of disk

accesses.

Some weird things about clockprobe (feedback requested!):

- Due to unexplained "glitches" in the vertical sync register, some timing

attempts go wrong.  this is detected in the program, and it tries again.

Does anyone know WHY those glitches are there, and how to circumvent them?

clockprobe measures the vertical refresh frequency, and then multiplies

that by the horizontal and verticalscreen size.

I can understand that some measurements may give multiples of the real

vertical sync time (because clockprobe could be time-sliced away in the

middle of a measurement). But some of them give LOWER results. e.g if the

vertical sync time is 14 msec, some measurements (about 1 in 200) give a

much smaller number (anything below 14 ms). This should be IMPOSSIBLE,

since I am using a hardware timer, which is unaffected by system load, and

you'd also expect the vertical screen rate to stay constant at 14 ms.

If you're interested, use the "-d" option, and see for yourself.

- has the tendency to over-estimate the clock rate by about 0.1 MHz. No real

clue why... (or is it just on MY machine?)